How It Was Built

- Electron was the desktop shell.

- The Electron main process used the serialport package to list COM ports, connect to the selected port, read telemetry, and write commands.

- The renderer was dashboard.html + styles.css + app.js.

- Three.js was used for the 3D accelerometer and gyroscope panels.

- The plots were not from Plotly or Chart.js; they were lightweight custom SVG plots drawn directly in JavaScript.

- The map was a Google Maps embed plus a direct “Open in Google Maps” link.

- The app was served inside Electron from a local http://127.0.0.1 server, not just opened as a raw file.

How Data Worked

- The dashboard expected a fixed telemetry schema:
TEAM_ID, MISSION_TIME, PACKET_COUNT, MODE, STATE, ALTITUDE, TEMPERATURE, PRESSURE, VOLTAGE, CURRENT, GYRO_R/P/Y, ACCEL_R/P/Y, GPS_TIME, GPS_ALTITUDE, GPS_LATITUDE, GPS_LONGITUDE, GPS_SATS, CMD_ECHO, CMD_ARG.

- Live data came in as newline-separated serial text over the selected COM port at 115200 baud.

- The parser accepted comma-separated or whitespace-separated rows, normalized headers, and rejected bad rows.

- A row was treated as valid only if packet count existed and enough numeric telemetry fields were present.

- Every valid row was pushed into an in-memory data[] array, given a sequence index, and then used to refresh the whole dashboard.

- Packets Lost was computed from gaps in PACKET_COUNT, not guessed.

- Export Log simply wrote the buffered data[] session to CSV.

- Logging Ready in that version meant “we are buffering data and can export it,” not “we are continuously saving to disk.”

How Each Visible Feature Was Implemented

- Status: latest row updated mission time, mode, state, packets received/lost, and GPS satellites.

- Quick Checks: simple health lights.
Telemetry Link meant serial connected.
Logging Ready meant at least one valid row exists.
Simulation Mode meant SIM state active/armed.
Battery OK only checked that voltage was present and parseable in that version.

- Command Panel: buttons sent plain-text commands like CAL, CX ON, CX OFF, SIM ENABLE, SIM ACTIVATE with a newline over serial.

- Last Sent was local UI state.

- Device Echo came from CMD_ECHO or immediate send feedback.

- Telemetry Cards: showed key values from the newest row, with light normalization for pressure and voltage.

- Plots: used the last ~80 samples, auto-scaled vertically, and showed latest values at the line end.

- Important nuance: even though the titles say “vs Time”, that build plotted mostly against sample/packet order, not a real time axis.

- Full Telemetry: rendered the full fixed telemetry schema into a grid from the newest packet.

- IMU 3D Graphs: showed raw accelerometer and gyroscope vectors as 3D arrows with a short history trail. It was a visualization of sensor vectors, not full attitude estimation.

- Location Mini Map: if latitude and longitude were valid, the app embedded Google Maps, marked GPS lock as active, and showed recent point summaries like Point A / Point B.

Simulation CSV

- This was the part evolving fastest during that CDR period.

- In the earlier March 17, 2026 build, CSV import was simpler and mainly extracted pressure samples.

- By the later April 16, 2026-era logic, the dashboard could normalize a simulation CSV into telemetry rows and replay them locally when no live serial link was connected.

- SIM ENABLE armed the loaded profile.

- SIM ACTIVATE started playback through the same widgets and plots.

- Playback delay was derived from MISSION_TIME, with limits so it stayed usable.

Yes, in your presentation context, these are basically the COTS software packages you used.

COTS means Commercial Off-The-Shelf:
software/tools/libraries that you did not build from scratch, but reused as existing solutions.

For your dashboard, the main COTS packages are:

- Electron

- SerialPort

- Three.js

And more generally also:

- Node.js / npm ecosystem

- built-in browser APIs used inside Electron

A clean way to explain it is:

“COTS packages are ready-made third-party software components we used instead of developing every layer ourselves. In our dashboard, the main COTS packages were Electron for the desktop app, SerialPort for USB telemetry communication, and Three.js for 3D IMU visualization.”